IN12-0201

DRAFT RESPONSE LANGUAGE

IN12-0201

IN12-0202

RE: Code Interpretation – NBIC Part 2, Section 5.2.2 through 5.2.3 Replacement of Stamping

Inquiry #1

If a National Board Commissioned Inspector has verified the replacement of stamped data or nameplate by an “R” Certificate holder on Corrugated rolls that are not stationary and subject to operation in multiple Jurisdictions, possibly by multiple owners, is the application of NB-136 “Replacement of Stamped Data Form” required for each of the multiple Jurisdictions?

Reply #1:

Yes, Section 5.2.1 clearly states that that requests for permission to re-stamp or replace nameplates shall be made to the Jurisdiction in which the pressure retaining item is installed, and that such application must be made on the NB-136 form.  No. If the vessel is not stationary, application should need only be made in the Jurisdiction where the nameplate is re-applied.

With the changes shown above the following question and reply can be deleted.

Inquiry #2

Can an NB-136 “Replacement of Stamped Data Form”, which is required to be signed by a National Board Commissioned Inspector, for Corrugated Rolls that are not stationary and subject to operation in multiple Jurisdictions, possibly by multiple owners, be filed with the National Board and copied to the applicable Jurisdiction in lieu of obtaining an approval signature from the Jurisdiction?

Reply #2:

No, Section 5.2.1 allows the action of the National Board in this manner only when there is no jurisdiction wherein the re-stamping or nameplate replacement is performed.